This file contains an overview of the important parts for the experimentation setup. It describes the necessary setup for the stepper motor, driver, Arduino and Pi and the procedure to control the setup. Pi information: https://www.raspberrypi.com/documentation/

- Hostname/username: microrobotpi

- Password: I<3microrobots

- Wifi: eduroam

- Password: tudmfmrs

- IP: 192.168.1.2

Arduino: https://docs.arduino.cc/hardware/giga-r1-wifi/

TMC2209: https://www.scribd.com/document/890126314/BIGTREETECH

# Wiring

Important to follow the wiring described in the circuit_schematic.pdf, including the capacitors for electronics protection. For powering the system, the following measures were taken:

- Arduino 	24V

- Raspberry Pi 	5.1V

- NEMA17 	28V

- Everything connected to the same ground

- On the Keysight, all channel grounds are connected to the earth terminal

- TMC2209 were referenced at 0.3V

# Control procedure

- Connect the laptop to the Pi (ethernet), the Pi to the master Arduino (USB-B to USB-C) and the master Arduino to the slave Arduinos (DB9+splitter)

- Power the Arduinos and Pi

- Connect only one motor to the TMC2209 and turn motor power on on the Keysight

- Wait a few seconds until connecting the second motor to the second TMC2209 (make sure the power channel for the motors stays in CV mode)

- Open the Terminal on the laptop (cmd in “Search”)

- Start the ssh connection inserting: ssh microrobotpi@microrobotpi.local

- If that does not work, try: ssh microrobotpi@192.168.12

- If that does not work, check if the connection is right

- Insert password (it will not be visible): I<3microrobots

- A connection will be established. To ensure connection, type ls to see the files on the Pi

- After uploading the four files in the folder “2. Experimental Setup/pi”, run server.py: python server.py

- On the laptop, open client_pi_path.py, insert the desired parameters and run it